Assignment No: 1

Name: Prasanna Khebade 	Roll No: 15 	PRN: 12311687 		Div: CS (AI) - B

Title: Setting up a small network (2PC/4 PC) and configuration for sharing resources.

Problem Statement: Setting up small wired computer network: Set up a small wired network of 2 to 4 computers using Hub/Switch/. It includes Preparation of Cables and setting up wired network.

Course Objective: To learn transmission mediums, networking devices and topologies used in the Internet.

Course Outcome: Select topology, essential components of physical layer and networking devices to design computer networks.

Tools Required: Computer, LAN Cards, RJ-45 Connectors, Hub/Switch, CAT-5 Cable, Cable tester, Crimping tool, etc.

Theory:

1. Introduction

Computer networks connect computers and devices to share data and resources. LANs link workstations (WS) and servers via cables; modems enable long‑distance communication by converting digital to analog signals and vice versa.

2. Network Devices

NIC connects PCs to the network (Arcnet, Ethernet, Token Ring).

A Hub is a central connector that broadcasts data to all connected devices; types include Passive, Active, and Intelligent.

A Bridge connects LAN segments using MAC addresses and reduces collisions.

A Switch sends data only to the intended device, is faster, and reduces collisions.

A Router connects different networks using IP addressing and selects the best path for data transmission.

3. Cabling & Pairing

CAT‑5 cable pairs are White/Orange, Orange, White/Green, Green, White/Brown, and Brown.

A Straight cable is used for Node to Switch connections; a Cross cable is used for Node to Node or Switch to Switch.

Crimping RJ‑45 involves stripping the cable, aligning wires, cutting evenly, inserting into the connector, crimping, and testing.

4. Transmission Media

Twisted Pair cable is inexpensive and reduces interference.

Coaxial cable has a shielded copper core and resists EMI.

Fiber Optic cable uses light for high‑speed, long‑distance data transfer.

Wireless media uses radio or microwave signals for flexible, mobile communication.

5. Topologies

Bus topology uses a single backbone cable, is low cost, but a single break stops the whole network.

Star topology connects each device to a central hub, easy to add or remove devices, but hub failure stops the network.

Ring topology connects devices in a circle, good for moderate traffic, but one failure can stop the network.

Mesh topology connects every node directly to every other, is reliable but expensive.

Tree topology connects star networks along a backbone, scalable but backbone failure disrupts all.

Hybrid topology combines two or more different topologies, offering flexibility but adding complexity.

Conclusion:

The cable was prepared with the correct T568B wiring sequence on both ends. The pairs were neatly arranged, and the crimping was secure.